Dr. Wolfgang Kabatzke

Hansastrasse 9

D-21502 Geesthacht

Deutschland / Germany

ECB-4PIO R02

Based on the design of the assembly jk82-PIO-WRAP by

Janich & Klass

Computersysteme

Neumarktstrasse 46

D-5600 Wuppertal 1

TECHNICAL DESCRIPTION

Content

1.	General characteristics of the ECB-4PIO-board	3

2.	IEI/IEO-Daisy-Chain	3

3.	Pin Assignments	3

3.1.	Pin assignment of parallel ports	3

3.2.	Bus Assignment and Fan Out/Fan In	5

4.	DIL-Switch Assignment	6

4.1.	Address settings of the I / O ports	6

4.2.	BAI/BAO-Daisy-Chain	7

4.3.	Default settings of the DIL-Switch	7

5.	BOM	8

6.	Differences between the ECB and 4PIO jk82-PIO-WRAP	8

6.1.	PIO Z80A	8

6.2.	LS-TTL IC	8

7.	Changes from R0.1 to R0.2	9

8.	Assembly Layout	9

9.	Wiring diagram	10

# General characteristics of the ECB-4PIO-board

The ECB-4PIO board contains 4 Z80A-PIO and the complete Bus-Control-Logic for the ECB-Systembus conform to KONTRON.

The technical data of the ECB-4PIO-board in brief:

- 4 Z80A-PIO 4

  - The PIO data signals and READY and STROBE are for each of the two data ports connected on a 26pin header. The pinout of this connector is compatible to jk82-PIO-WRAP by Janich & Klass.

- Single Eurocard with ECB-bus

  - ECB-4PIO is full IM2-interruptible. The bus control logic allows RETI-detection for the internal PIO. IEI and IEO are wired to look ahead. Even with a partial assembly the IEI/IEO Daisy Chain is fully preserved.

- Power consumption at typical 4MHz 400mA (at 5V)

# IEI/IEO-Daisy-Chain

The four PIO are able to realize vectorinterrupt and are prioritized in a daisy chain. The daisy chain is connected to a carry-look-ahead logic, so that the processing delay is in the range of 10 to20ns.

BUS    --->    PIO 0    --->    PIO 1	---->	PIO 2    --->    PIO 3    ---->    BUS

IEI		    IEO  IEI            IEO   IEI 	IEO   IEI	    IEO

Through pull-up resistors of the IEO PIO-1, PIO-2 and PIO-3 is a proper function of the daisy chain be guaranteed even if not all 4 PIO are fitted.

In the case of assembly with only one PIO is absolutely PIO-0 to assemble! In the case of assembly with only two PIO are absolutely PIO-0 and PIO-1 to assemble! In the case of assembly with only three PIO are absolutely PIO-0, PIO-1 and PIO-2 to assemble!

In all other cases, the daisy chain is broken!

# Pin Assignments

## Pin assignment of parallel ports

Each PIO is a 26-pin connector associated with the following assignment:

Signal	Pin		Pin		Signal

B7		1		2		A7

B6		3		4		A6

B5		5		6		A5

B4		7		8		A4

B3		9		10		A3

B2		11		12		A2

B1		13		14		A1

B0		15		16		A0

+5V	17		18		GND

BSTB	19		20		ASTB

BRDY	21		22		ARDY

NC		23		24		NC

NC		25		26		NC

This pinout is compatible to jk82-PIO-WRAP by Janich & Klass.

## Bus Assignment and Fan Out/Fan In

Input/Output	|			LS-Fan	out	|	in

A0			|	Address 0			--	|	1

A1			|	Address 1			--	|	1

A2			|	Address 2			--	|	1

A3			|	Address 3			--	|	1

A4			|	Address 4			--	|	1

A5			|	Address 5			--	|	1

A6			|	Address 6			--	|	1

A7			|	Address 7			--	|	1

|						|

D0			|	Data 0				60	|	1

D1			|	Data 1				60	|	1

D2			|	Data 2				60	|	1

D3			|	Data 3				60	|	1

D4			|	Data 4				60	|	1

D5			|	Data 5				60	|	1

D6			|	Data 6				60	|	1

D7			|	Data 7				60	|	1

|						|

/IORQ		|	I/O-Request (low)		--	|	1

/RD		|	Read (low)			--	|	1

/M1		|	Machine Cyvcle 1 (low)	--	|	1

Ф			|	Clock				--	|	1

/PWCLR		|	Power on clear (low)	--	|	1

/INT		|	Interrupt (low)		3,2mA	|	--

IEI			|	Interrupt Enable In (high)	--	|	1

IEO		|	Interrupt Enable Out (high)19	|	--

/BAI		|	Bus-Ack In (low)		--	|	--

/BAO		|	Bus-Ack Out (low)		--	|	--

Assignment of the VG96 connector P1 (ECB-bus to Kontron)

a	b	c

+5V	1		1	+5V

D5		2		2	D0

D6		3		3	D7

D3		4		4	D2

D4		5		5	A0

A2		6		6	A3

A4		7		7	A1

A5		8		8

A6		9		9	A7

10		10

11		11	IEI

/BAI	12		12

- 13

14		14	D1

15		15

16		16	IEO

/BAO	17		17

- 18

- 19

/M1	20		20

21		21	/INT

22		22

23		23

24		24	/RD

25		25

26		26	/PWCLR

/IORQ	27		27

28		28

29		29	Ф

30		30

31		31	/RESET

GND	32		32	GND

# DIL-Switch Assignment

DIL-Switch-Assignment S1

4-5	x	x	A4

3-6	x	x	A5

2-7	x	x	A6

  - x	x	A7

## 4.1.	Address settings of the I / O ports

The ECB-4PIO board occupies 16 I / O addresses, the upper 4 bits of the address are set by DIP switches for A4 to A7 (see assignment of S1)

The address assignment looks like this:

A7	A6	A5	A4	A3	A2	A1	A0	(Hex)	Funktion

x	x	x	x	0	0	0	0	(20)	PIO 0 DATA	Kanal A

x	x	x	x	0	0	0	1	(21)	PIO 0 CTRL	Kanal A

x	x	x	x	0	0	1	0	(22)	PIO 0	DATA	Kanal B

x	x	x	x	0	0	1	1	(23)	PIO 0 CTRL	Kanal B

x	x	x	x	0	1	0	0	(24)	PIO 1 DATA	Kanal A

x	x	x	x	0	1	0	1	(25)	PIO 1 CTRL	Kanal A

x	x	x	x	0	1	1	0	(26)	PIO 1	DATA	Kanal B

x	x	x	x	0	1	1	1	(27)	PIO 1 CTRL	Kanal B

x	x	x	x	1	0	0	0	(28)	PIO 2 DATA	Kanal A

x	x	x	x	1	0	0	1	(29)	PIO 2 CTRL	Kanal A

x	x	x	x	1	0	1	0	(2A)	PIO 2	DATA	Kanal B

x	x	x	x	1	0	1	1	(2B)	PIO 2 CTRL	Kanal B

x	x	x	x	1	1	0	0	(2C)	PIO 3 DATA	Kanal A

x	x	x	x	1	1	0	1	(2D)	PIO 3 CTRL	Kanal A

x	x	x	x	1	1	1	0	(2E)	PIO 3	DATA	Kanal B

x	x	x	x	1	1	1	1	(2F)	PIO 3 CTRL	Kanal B

The information in the column "(Hex)" refer to the default address settings (see 4.3)!

For address setting is important to note that "ON" a zero and "OFF" means a one (The switches are set to GND!).

## 4.2.	BAI/BAO-Daisy-Chain

The Busaknowledge-Daisy-Chain BAI/BAO is on the ECB-4PIO static connected. Since this PCB only for the ECB bus (compatible to) KONTRON was developed, the implementation is static.

## 4.3.	Default settings of the DIL-Switch

On delivery of an assembled circuit board, the DIP switches are set as follows:

4-5		3-6		2-7		1-8

„ON“,		„OFF“,		„ON“,		„ON“

A4		A5		A6		A7

That means I / O base address of the board: 20H

# BOM

IC	1		74LS07			Socket:	3 x 14pin

IC	2		74LS08					2 x 16pin

IC	3		74LS27					2 x 20pin

IC	4		74LS541					4 x 40pin

IC	5		74LS85

IC	6		74LS245

IC	7		74LS139

IC	8, 9, 11,12	Z80A-PIO

R			12 x 	4k7

C			12 x 	100nF	 RM 2,5

S 1		DIL-Switch 8pin

S 2		DIL-Switch 4pin

P	1		VG-96 connector	(a,b,c)

P	2-5		Header  4 x 26pin	(2 x 13pin)

# Differences between the ECB and 4PIO jk82-PIO-WRAP

Between the two PCB are differences, not logically, but in design.

## PIO Z80A

There layout of ECB-4PIO requires a rearrangement of IC8 - IC10 or PIO-0 - PIO-3 (jk82-PIO-WRAP) to IC8 to PIO, 11, 9, 12 and PIO-0 to PIO-3:

Jk82-PIO-WRAP				ECB-4PIO

IC8	PIO-0					IC8	PIO-0

IC9	PIO-1					IC11	PIO-2

IC10	PIO-2					IC9	PIO-1

IC11	PIO-3					IC12	PIO-3

The order in the daisy chain is amended IC numbers are not influenced by the rearrangement.

## LS-TTL IC

By the replacement of some component types and the use of a new IC with open collector (IC10), Component numbering was reassigned:

Jk82-PIO-WRAP				ECB-4PIO

IC1	74LS85				IC5	74LS85

IC2	74LS139				IC7	74LS139

IC3	74LS21				IC10	74LS21

IC4	74LS08				IC2	74LS08

IC5	74LS27				IC3	74LS27

IC6	74LS243				IC4	74LS541

IC7	74LS245				IC6	74LS245, 74LS645

IC1	74LS07

This was necessary because the driver IC6 (74LS243) of the circuit of jk82-PIO-WRAP by IC4 (74LS541) has been replaced. So now the AB2, AB3, IEI (by bus) and Ф (by bus) driven, as these signals are shared.

Furthermore, all /INT-outputs of PIO-0 PIO-3 are coupled via a resistor to +5 V (due to open-drain) and a driver of IC1 (74LS07) decouples the PIO from the ECB (Now are all PIO undriven with any signal on the ECB bus).

The DIL switch for the bridge /BAI to /BAO of jk82-PIO-WRAP is on the ECB-4PIO replaced by a fixed bridge.

The rest of the circuit was not changed. Es There is simply no longer WRAP box.

# Changes from R0.1 to R0.2

New is in R0.2 the DIP-Switch S2. With the help of S2 we can use the KONTRON-conform Reset-Signal /PWRCLR (ECB C26) as RESET for PIO or like N8VEM the Signal /RESET (ECB C31)

# Assembly Layout

.

# Wiring diagram